EMM 305: METROLOGY 						20TH July 2023

Continuous Assessment Test

Questions

- A clearance fit has to be provided for a shaft and bearing assembly having a diameter of
40mm. Tolerances on hole and shaft are 0.006 and 0.004 mm, respectively. The tolerances are
disposed unilaterally. If an allowance of 0.002mm is provided, find the limits of size for hole
and shaft when

(a) hole basis system and

(b) shaft basis system are used							(5 Marks)

- State the requirements that the material used to manufacture gauges should satisfy.

(5 marks)

- Define a comparator and give THREE classification of comparators.		(5 marks)

- With the help of a neat sketch, explain the functional parts of a dial indicator.      (4Marks)

- Discuss the various metrological applications of an optical projector		(4 Marks)

- With the help of an illustration, explain the following terms: roughness, waviness, lay, and

flaws 										(7 Marks)